| Image Device |  |  | 1/3” Color CCD Super HAD II (Sony Chipset) effioA |
| --- | --- | --- | --- |
| Effective Picture Elements |  |  | NTSC: 976x494 PAL: 976x582 |
| Horizontal Resolution |  |  | 800 TVL |
| Minimum Illumination |  |  | 0.01ux/ F1.2 (Day) |
| Day to Night Illumination |  |  | Under 5 Lux |
| S/N Ratio |  |  | More than 50dB (AGC OFF) |
| Shutter Speed Control |  |  | NTSC:1/60~1/100,000, PAL:1/50~1/100,000 |
| Lens Furnished |  |  | Vari-Focal Lens with ICR / board lens |
| Auto Iris Control |  |  | DC Driver |
| Gain Control |  |  | Auto/ Manual |
| White Balance |  |  | ATW / PUSH / USER1 / USER2 / ANTI CR / Manual / PUSH LOCK |
| ATR |  |  | ON/ OFF |
| DNR |  |  | ON/ OFF |
| Back Light Compensation |  |  | ON/ OFF |
| High Light Compensation |  |  | ON/ OFF |
| PRIVACY |  |  | Maximum 8 Areas |
| MOTION DET |  |  | 4 Areas |
| Camera ID |  |  | Up to 52 Characters |
| Mirror |  |  | ON/ OFF |
| Positive/ Negative |  |  | ON/OFF |
| Video Output |  |  | 1Vp-p/ 75 Ohms |
| Power Supply |  |  | DC 12V±10% |
| Power Consumption | Vari-Focal Lens |  | 120mA max. |
|  | Vari-Focal Lens+IR | IR On | 350mA max. |
|  |  | IR Off | 120mA max. |
| Operating Temp. |  |  | -10℃ ~ 50 ℃ (14 to 122℉) |
| Gamma Characteristic |  |  | 0.45 |
| Synchronous System |  |  | INT |
| Dimensions (WxD) |  |  | 32 mm x32 mm (Camera board) 37 mm x35 mm (OSD board) 110mm x 82 (H) mm |

RYK2G004