| Image Device | 1/3” SONY COLOR Super HAD CCDⅡ (Sony Chipset) effioA |
| --- | --- |
| Effective Picture Elements | NTSC: 976x494 PAL: 976x582 |
| Horizontal Resolution | 800 TVL |
| Minimum Illumination | 0.01 Lux/F1.2 (DAY) |
| Day to Night Illumination | Under 5 Lux |
| S/N Ratio | More than 50dB (AGC OFF) |
| Shutter Speed Control | Auto:NTSC1/60~1/100,000s,PAL1/50~1/100,000s Manual: 1/60, 1/100, 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/10000 |
| Lens Furnished | Vari-Focal Lens with ICR |
| Auto Iris Control | DC Driver |
| Gain Control | Auto/ Manual |
| White Balance | ATW / PUSH / USER1 / USER2 / ANTI CR / Manual / PUSH LOCK |
| ATR | ON/ OFF |
| DNR | ON/ OFF |
| Back Light Compensation | ON/ OFF |
| High Light Compensation | ON/ OFF |
| PRIVACY | Maximum 8 Areas |
| MOTION DET | 4 Areas |
| Camera ID | Up to 52 Characters |
| Mirror | ON/ OFF |
| Positive/ Negative | ON/OFF |
| Video Output | 1Vp-p/ 75 Ohms |
| Infrared Luminary | 42 pieces IR- LED |
| Wavelength | 850 nm. |
| Illuminate Distance | Up to 40m, Low / Mid / High switchable |
| Power Supply | DC 12V±10%, Dual Power DC12V/AC24V (Optional) |
| Power Consumption | IR ON (Low): 320mA (Mid): 420mA (High):800mA IR OFF: 130mA @DC12V Max |
| Operating Temp. | -10℃ ~ 50 ℃ (14 to 122℉) |
| Gamma Characteristic | 0.45 |
| Synchronous System | INT |
| Dimensions (WxD) | 32 mm x32 mm (Camera board)37 mm x35 mm (OSD board) 95 mmX267(L)mm –excluding sunshield |

RYK2W004