HTTP Command Description

(2013/06/21)

Syntax: http://{Server IP}/{command}?{argument1}&{argument2}&{argument3}&{..}....

The braces “{}”in the above syntax indicates a command or parameter. When order a command, there must be an “&” between commands and parameters.(ID=64 bytes ID string getting from login command )

P.S.: If your ID is wrong, NVR returns html with error string”Authentication Error:Access Denied, authentication error”,and please pay attention to what returned value means.

Note:

All HTTP commands list in this document are compatible for NVR, DVR & HDVR.

But there are some exceptions! And they are marked in red font.

Ex: [ DVR ONLY ] , [ DVR, HDVR ONLY ] [ NVR ONLY ]

- Purpose: Obtain login information and register a ID

| Syntax | http://{Server IP}/auth/login |
| --- | --- |
| Parameters | None |
| Example | Obtain login information. - ID: A 64 bytes thread id string. - MODEL_ID: Model ID. - BRAND_ID: Brand ID. - CHANNELS: Total camera number. - TOTAL_IP_CAM : Total IP camera number - VIDEO_TYPE : NTSC (13 scan lines), PAL(16 scan lines) - DOME_PROTOCOL : 0 = None , 1 = DSCP, 2 = AD422, 3 = PELCO D , 4 = PELCO P , 5 = Fastrax 2, 6 = Molynx, 7 = JVC, 8 = Panasonic_C, 9 = Panasonic_N, 10 = GE RS-485, 11 = GE RS-422, 12 = Samsung E - DOME_PROTOCOL_NAME: All dome protocol option from OSD. - DOME_ID: Each channel’s dome ID. - CAMERA_TITLE: All camera title of cameras. - TOTAL_PPS: Total PPS of all videos. - Date Display Mode: 0=Y/M/D , 1=M/D/Y , 2=D/M/Y. - Time Display Mode: 0=12hr, 1=24hr. - Date/Time Order: 0=date first, 1=time first. - MJPEG: Motion JPEG model. (Not in use for NVR model) - TRANSCODER: Transcoder recording mode(Not in use for NVR model) - AUDIOSTATE: 0=none, 1=left, 2=right, 3=all. - TEXTSTATE: 0=not have POS module, 1=have POS module - Username: The name which NVR user used to login. - HA1: The encrypted info of <account><realm><password> - MODEL_PREVIX: The prefix of model name. - SW_VER: NVR software version. - HW_VER: NVR’s hardware version. P.S.: If connecting full, it returns “connecting full”. If admin is online, it returns “There is another administrator online”. Thread id will be registered in this command. |

- Purpose: Obtain login information

| Syntax | http://{Server IP}/auth/loginInfo.get |
| --- | --- |
| Parameters | None |
| Example | Obtain login information. - MODEL_ID: Model ID. - BRAND_ID: Brand ID. - CHANNELS: Total camera number. - TOTAL_IP_CAM: Total IP camera number. - VIDEO_TYPE : NTSC (13 scan lines), PAL(16 scan lines) - DOME_PROTOCOL : 0 = None , 1 = DSCP, 2 = AD422, 3 = PELCO D , 4 = PELCO P , 5 = Fastrax 2, 6 = Molynx, 7 = JVC, 8 = Panasonic_C, 9 = Panasonic_N, 10 = GE RS-485, 11 = GE RS-422, 12 = Samsung E - DOME_PROTOCOL_NAME: All dome protocol option from OSD. - DOME_ID: Each channel’s dome ID. - CAMERA_TITLE: All camera title of cameras. - TOTAL_PPS: Total PPS of all videos. - Date Display Mode: 0=Y/M/D , 1=M/D/Y , 2=D/M/Y. - Time Display Mode: 0=12hr, 1=24hr. - Date/Time Order: 0=date first, 1=time first. - MJPEG: Motion JPEG model. (Not in use for NVR model) - TRANSCODER: Transcoder recording mode(Not in use for NVR model) - AUDIOSTATE: 0=none, 1=left, 2=right, 3=all. - TEXTSTATE: 0=not have POS module, 1=have POS module. - Username: The name which NVR user used to login. - HA1: The encrypted info of <account><realm><password>. - MODEL_PREFIX: The prefix of model name. - SW_VER: NVR software version. - HW_VER: NVR’s hardware version. |

- Purpose: Connect to the server

| Syntax | http://{Server IP}/index |
| --- | --- |
| Parameters | None |
| Example | Connect to the server |
|  | http://{ServerIP}/index Return: the home page displays |

- Purpose: Get video stream

| Syntax | http://{ServerIP}/video.vdo?action=live&ch{ID}=0,1,2&CO=xxx&THREAD_ID=ID 0: disable stream, 1:enable main stream, 2: enable dual stream Each bit of xxx represent the covert channel, ex: bit N-1 for channel N) |
| --- | --- |
| Parameters | Channel ID, Covert unblocking setting, Thread ID |
| Example 1 | Request video stream from channel 1, 2 |
|  | http://{ServerIP}/video.vdo?action=live&ch1=1&ch2=2&THREAD_ID=ID Return: If success, it returns a main video stream of ch1 and dual stream of ch2. |
| Example 2 | Request video stream from channel 1, 2 which are covert at local NVR. |
|  | http://{ServerIP}/video.vdo?action=live&ch1=1&ch2=1&CO=3&THREAD_ID=ID Return: - If success, it returns a video stream of ch1 and ch2. - Though channel 1 & 2 is set to covert at local DVR, the data will still be transmitted to client by the argument CO=3. |

- Purpose: Get Information on duration of recorded video for playback, total channels or camera titles

| Syntax | http://{Server IP}/info. get?{argument} |
| --- | --- |
| Parameters | SW_VER; To obtain S/W info.. HW_VER; To obtain H/W info.. DeviceInfo; To obtain the info of NVR. CAMERA_TITLE; To obtain the camera title.. PBTime; To obtain record duration.. RelayOutStatus; To obtain relay out status.. Livetime; To obtain current live time info.. CHANNELS; To obtain the total number of cameras.. FORMAT_PERCENTAGE; To obtain the percentage of format process.. CAMERA_COVERT; To obtain the covert info.. CFGSIZE; To obtain the size of config file. [Unit in byte] .. SETMOTION_0; To obtain motion zone info.. SETPRIVACYMASK_0; To obtain the privacy mask.. |
| Example 1 | To obtain S/W info. |
|  | http://{ServerIP}/info.get?SW_VER Return: SW_VER=[00246-0332-00734-09515] |
| Example 2 | To obtain H/W info. |
|  | http://{ServerIP}/info.get?HW_VER Return: HW_VER=[A4CB-1007-9A40-UB15] |
| Example 3 | To obtain the camera info of NVR |
|  | http://{ServerIP}/info.get?DeviceInfo Return: MODEL_ID=[144] BRAND_ID=[10] CHANNELS=[8] TOTAL_IPCAM_NUM=[8] |
| Example 4 | To obtain the camera title. |
|  | http://{ServerIP}/info.get?CAMERA_TITLE Return: CAMERA_TITLE=[Ch1,Ch2,Ch3,Ch4,Ch5,Ch6,Ch7,Ch8,] |
| Example 5 | To obtain record duration |
|  | http://{ServerIP}/info.get?PBTime Return: StartTime / EndTime : Local time.(Time info with DST and Timezone) RealStartTime / RealEndTime : Real time.(Time info without DST and Timezone) StartTime=[1363973296] ,Fri, 22 Mar 2013 17:28:16.,514c94b0 EndTime=[1364407773] ,Wed, 27 Mar 2013 18:09:33.,515335dd RealTimeSupport=[1] RealStartTime=[1363973296] RealEndTime=[1364407773] TimeZoneOffset=[0] DSTEnable=[0] DSTStartMonth=[2] DSTStartWeek=[4] DSTStartWDay=[0] DSTStartHour=[1] DSTStartMin=[0] DSTEndMonth=[9] DSTEndWeek=[4] DSTEndWDay=[0] DSTEndHour=[1] DSTEndMin=[0] DSTBias=[60] DSTStartTime=[1364691600] DSTEndTime=[1382832000] DSTLastYearStartTime=[1332637200] DSTLastYearEndTime=[1351382400] DLTBoundary=[0] RealDLTBoundary=[0] |
| Example 6 | To obtain relay out status |
|  | http://{ServerIP}/info.get?RelayOutStatus Return: TotalRelayOut=[3], The total number of relay-out. RelayOutA=[0],0 is Off, 1 is On. RelayOutB=[0], 0 is Off, 1 is On. RelayOutC=[0] 0 is Off, 1 is On. |
| Example 7 | To obtain current live time info. |
|  | http://{ServerIP}/info.get?Livetime Return: UTCtime=[1366600043] TimeZoneOffset=[32400] DSTEnable=[0] DSTStartMonth=[2] DSTStartWeek=[1] DSTStartWDay=[0] DSTStartHour=[2] DSTStartMin=[0] DSTEndMonth=[10] DSTEndWeek=[0] DSTEndWDay=[0] DSTEndHour=[2] DSTEndMin=[0] DSTBias=[60] Livetime=[2013/04/22 12:07:23] |
| Example 8 | To obtain the total number of cameras. |
|  | http://{ServerIP}/info.get?CHANNELS Return: CHANNELS=[20] IPCAMERAS=[8] It means 12 analog cameras and 8 IP cameras. |
| Example 9 | To obtain the percentage of format process. |
|  | http://{ServerIP}/info.get?FORMAT_PERCENTAGE Return: FORMAT_PERCENTAGE=[0] |
| Example 10 | To obtain the covert info. |
|  | http://{ServerIP}/info.get?CAMERA_COVERT Return: (Ex: for 8 channels NVR, 1 is covert, and 0 is not.) CAMERA_COVERT=[1,0,0,0,0,0,0,0]. |
| Example 11 | To obtain the size of config file.[Unit in byte] |
|  | http://{ServerIP}/info.get?CFGSIZE Return: CFGSIZE=[14936] |
| Example 12 | To obtain motion zone info. |
|  | http://{ServerIP}/info.get?SETMOTION_{channel_id} [ Get info failed ] Return -1; Because of both local/remote side are in the same page. Return -2; Because of the channel argument is out of range. [ Get info success ] Return the motion zone of all channels. The motion zone will be represented in 12x16. Bit-wise mapped motion zone info. Only the lower 16 bits are used. Each bit represents a motion block. 0x1 for motion block selected, and 0x0 for motion zone deselected. From highest bit to lowest bit will represent each row of motion zone from the right to the left. Ex: http://{ServerIP}/info.get?SETMOTION_1 The following replied message represents the ch1 motion zone is deselected. And ch2 motion zone is selected since the lower 16 bits are marked as 0x1. -214748364810 => 0x80000000 -214741811310 => 0xFFFFFFFF8000FFFF SETMOTION=[-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147483648,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,-2147418113,…] |
| Example 13 | To obtain the privacy mask on analog channels. [ DVR, HDVR ONLY ] |
|  | http://{ServerIP}/info.get?SETPRIVACYMASK_{channel_id} [ Get info failed ] Return -2; Because of the query command is wrong. [ Get info success ] Return the privacy mask of all channels. The privacy mask will be represented in 12x16. Bit-wise mapped info for each row of privacy mask. 0x1 for privacy block enabled, 0x0 for privacy block disabled. From highest bit to lowest bit will represent the privacy block from the right to the left. Ex: http://{ServerIP}/info.get?SETPRIVACYMASK_1 The following returned sequence will represent the privacy masks of all channels. And the privacy mask has been applied to the channel #1. SETPRIVACYMASK=[65408,65408,65408,65408,65535,65535,65535,65535,65535,65535,65535,65535,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,] |

- Purpose: Obtain the configuration of server

| Syntax | http://{Server IP}/auth/config.get |
| --- | --- |
| Parameters | None |
| Example | Get configuration |
|  | http://{ServerIP}/auth/config.get Return: Ex: |

- Purpose: Set configuration with specific argument.

| Syntax | http://{Server IP}/auth/config.set?{parameter}&{THREAD_ID} |
| --- | --- |
| Example1 | Set channel returned configuration in certain language |
|  | Ex: http://{ServerIP}/auth/config.set?PCLanguage=1&THREAD_ID=ID Return: Language=[1] Success 0; English; 13; Swedish. 1; Chinese[Traditional]; 14; Portugal. 2; Japanese; 15; Hungarian. 3; Finnish; 16; Slovak. 4; Bulgarian; 17; Greece. 5; Russian; 18; Persian. 6; Polish; 19; Chinese [Simplified]. 7; Spanish; 20; Norwegian. 8; German; 21; Turkish. 9; Italian; 22; Czech. 10; French; 23; Hebrew. 11; Dutch; 24; Thai. 12; Danish. |
| Example2 | Set sequence. |
|  | http://{ServerIP}/auth/config.set?SETSEQ{MAIN1, MAIN2,CALL1,CALL2,VIEW1,VIEW2,VIEW3 or VIEW4}=sequence channel order&THREAD_ID=ID The channel number is from 0 to 15. Ex: http://127.0.0.1/auth/config.set?SETSEQMAIN1= 0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,&THREAD_ID=ID Return: If Success, “<p>Set sequence Success</p>” is returned. If failed, “<H2>Set sequence Failed</H2>” is returned. |
| Example3 | Send operation command to HDD. [Ex: ADD, REMOVE…etc] |
|  | Format: http://{ServerIP}/auth/config.set?{HardDisk}={Operation}&THREAD_ID=ID HardDisk. INTERNALHDD; Internal disk. EXTERNALHDD; External disk. Operation. 1; Add disk. 2; Remove disk. 3; Format disk. 5; Low format disk. Ex: http://{ServerIP}/auth/config.set?INTERNALHDD=2&THREAD_ID=ID Return: This example is to remove internal disk. If success, “Success” is returned. If failed, “Failed” is returned. |
| Example 4 | Reboot NVR. |
|  | Ex: http://{ServerIP}/auth/config.set?REBOOT&THREAD_ID=ID Return: NVR rebooting. |
| Example 5 | To set up user account. |
|  | Ex: http://{ServerIP}/auth/config.set?ACCOUNTSETUP_ACCOUNT[idx]=account&THREAD_ID=ID idx represents the order list by issuing cmd /auth/account.get, |
| Example 6 | To set up user password. |
|  | Ex: http://{ServerIP}/auth/config.set?ACCOUNTSETUP_PASSWORD1[idx]=password&THREAD_ID=ID idx represents the order list by issuing cmd /auth/account.get, |
| Example 7 | To set up user privilege. |
|  | Ex: http://{ServerIP}/auth/config.set?ACCOUNTSETUP_PRIVILEGE[idx]=level&THREAD_ID=ID idx represents the order list by issuing cmd /auth/account.get, and level is the privilege level from 0 to 8 where 0 stands for Disable. |
| Example 8 | To set privilege level for certain menus. |
|  | Ex: http://{ServerIP}/auth/config.set?Permissions_Setup[idx]=level&THREAD_ID=ID where idx is the order list by issuing cmd /auth/permission.get level is the privilege level from 0 to 8 where 0 stands for Disable. |
| Example 9 | To obtain DDNS domain name. |
|  | Ex: http://{ServerIP}/auth/config.set?DDNS_DOMAINNAME&THREAD_ID=ID Return: DDNS_DOMAINNAME=[DDNS domain name] |
| Example 10 | To update DDNS hostname. |
|  | Ex: http://{ServerIP}/auth/config.set?HOST_NAME_UPDATE&THREAD_ID=ID Return: The host name registered in the DDNS server! |
| Example 11 | To update Motion Detect option’s drop-down list.NVR will reply the correct drop-down list. Either On/Off or Day / Nigh t /Weekend / …etc. |
|  | Ex: http://{ServerIP}/auth/config.set?MOTIONDET_CHG&THREAD_ID=ID Return: VALUE=[0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,]ITEMS=[ OFF, Day, Night, Day & Night, Weekend, Day & Weekend, Night & Weekend, All,]ITEM_COUNTS=[20]TYPE=[STR]MIN=[0]MAX=[7]DELTA=[1]ENABLE=[1]UNIT=[null]LANGNAME=[移動偵測]LANGITEMS=[ 關, 日間, 夜間, 日間&夜間, 週末, 日間&週末, 夜間&週末, 全部,] |
| Example 12 | To obtain the result of ezRecord. |
|  | Ex: http://{ServerIP}/auth/config.set?EZRECORD_OPEN&THREAD_ID=ID Return: (Estimate_RecDay, RecDay_Min, RecDay_Max) = (1, 0, 6) - Estimate_RecDay = 1 -> How Many Days to Record = 1 day ( PPSLevel_ 1, 3.75, 7.5, 11.3, 15, 30 ) = ( 2_ 1, 3.75, 7.5, 11.3, 15, 30 ) - PPSLevel = 2 -> Average Normal PPS = 7.5 (QualityLevel _ Low, Fair, Mid, High, Best) = (3 _ Low, Fair, Mid, High, Best) - QualityLevel = 3 -> Average Normal Quality = High |
| Example 13 | To set privacy masks on analog channels. [ DVR, HDVR ONLY ] |
|  | Format: http://{ServerIP}/auth/config.set?SETPRIVACYMASK_{channel}=mask&THREAD_ID=ID Ex: http://{Server_IP}/auth/config.set?SETPRIVACYMASK_3=65408,65408,65408,65408,65535,65535,65535,65535,65535,65535,65535,65535&&THREAD_ID=ID Set privacy mask on channel 3. 6540810 = 0xFF80, 6553510 = 0xFFFF; Bit-wise mapping representation for each row of the privacy mask.0x1 for covert block and 0x0 for non-covert block. From the highest bit to the lowest bit represents each block in a row from the right to the left. 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 1; 1. 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 1; 1. 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 1; 1. 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 1; 1. 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1. 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1. …; …; …; …; …; ..; …; …; …; …; …; …; …; …; …; …. 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1. |
| Example 14 | To set motion zone. |
|  | Format: http://{ServerIP}/auth/config.set?SETMOTION_{channel}=motion_mask&THREAD_ID=ID Ex: http://{ServerIP}/auth/config.set?SETMOTION_1=65408,65408,65535,65535,65507,65507,65507,65507,65535,65535,65535,65535,&&THREAD_ID=ID Set privacy mask on channel 3. 6540810 = 0xFF80, 6553510 = 0xFFFF; Bit-wise mapping representation for each row of the motion mask.0x1 for motion detect block and 0x0 for non-motion detect block. It’s similar to the representation of privacy mask! |
| Example 15 | To turn off motion bar indicator at the remote-side. |
|  | Format: http://{ServerIP}/auth/config.set?MOTIONBAR_CLOSE&THREAD_ID=ID |
| Example 16 | To setup the simultaneous output as No or Yes. [ DVR, HDVR ONLY ] |
|  | Format: http://{ServerIP}/auth/config.set?Simultaneous_Output={OnOff}&&THREAD_ID=ID OnOff: 1 for on and 0 for off. Return: None. And DVR will reboot to apply the change. |
| Example 17 | To setup analog camera name. ONLY English is supported. |
|  | Format: http://{ServerIP}/auth/config.set?Camera_Name[idx]=Name&&THREAD_ID=ID idx is the camera index which start from 0. Ex: http://{ServerIP}/auth/config.set?Camera_Name[2]=Lobby&&THREAD_ID=ID To setup camera 3’s name as “Lobby”. |
| Example 18 | To setup IP camera name. ONLY English is supported. |
|  | Format: http://{ServerIP}/auth/config.set?IP_Camera_Name[idx]=Name&&THREAD_ID=ID idx is the camera index which start from 0. Ex: http://{ServerIP}/auth/config.set?IP_Camera_Name[16]=Kitchen&&THREAD_ID=ID To setup IP camera 17’s name as “Kitchen”. |
| Example 19 | To sync NTP (Network Time Protocol) server. |
|  | Format: http://{ServerIP}/auth/config.set?Manually_Time_Sync=1&THREAD_ID=ID |
| Example 20 | To set alert notification for alert set 1, 2, 3. |
|  | Format: http://{ServerIP}/auth/config.set?Alert_Notification[idx]={value}&THREAD_ID=ID idx is the alert set index, start from 0 to 2. value 0x1 for Event Notification, 0x2 for Alive Notification, 0x3 for Both. Ex: http://{ServerIP}/auth/config.set?Alert_Notification[2]=3&THREAD_ID=ID Set Alert set#3 as both Event Notification & Alive Notification. |
| Example 21 | To set alert IP for alert set 1, 2, 3. |
|  | Format: http://{ServerIP}/auth/config.set?Alert_IP[idx]={IP}&THREAD_ID=ID idx is the alert set index, start from 0 to 2. IP is the IP address. Ex: http://{ServerIP}/auth/config.set?Alert_IP[2]=192.168.5.123&THREAD_ID=ID Set Alert IP as 192.168.5.123 on set#3. |
| Example 22 | To set alert port for alert set 1, 2, 3. |
|  | Format: http://{ServerIP}/auth/config.set?Alert_Port[idx]=port&THREAD_ID=ID idx is the alert set index, start from 0 to 2. Ex: http://{ServerIP}/auth/config.set?Alert_Port[0]=1111&THREAD_ID=ID Set Alert set#1 alert port as 1111. |
| Example 23 | Set monitor’s resolution |
|  | Format: http://{ServerIP}/auth/config.set?VGA_Resolution=value&THREAD_ID=ID value; Resolution. 0; 1024x768. 1; 1280x1024. 2; 1280x720(720P). 3; 1920x1080(1080P). |
| Example 24 | Set Event/Alarm Selection in E-mail Management / Event Backup Setup. |
|  | Format: http://{ServerIP}/auth/config.set?multi_list{Index1}_item{Index2}[Set]=YesNo&THREAD_ID=ID YesNo. 0; Yes. 1; No. Set. 0; E-mail #1. 1; E-mail #2. 2; E-mail #3. Index1; Menu. 0; E-mail Management. 1; Event Backup Setup. The meaning of Index2. Index2; with UPS support; without UPS support. 0; Motion Detect Event; Motion Detect Event. 1; Video Loss Event; Video Loss Event. 2; Alarm Trigger Event; Alarm Trigger Event. 3; UPS Power Failure Alarm; System Fan Alarm. 4; System Fan Alarm; Disk Error Alarm. 5; Disk Error Alarm; (Not in used). Ex: http://{ServerIP}/auth/config.set?multi_list0_item0[0]=0&multi_list0_item1[0]=1&THREAD_ID=ID This command is to set E-mail Set#1’s Motion Detect Event & Video Loss Event as No & Yes, respectively. (Index2=0,1; Index1=0, Set=0) Ex2: Event Backup Setup only has 1 set of Event/Alarm setting. http://{ServerIP}/auth/config.set?multi_list1_item0=0&multi_list1_item1=1&THREAD_ID=ID This command is to set Event Backup Setup’s Event/Alarm Selection Motion Detect Event & Video Loss Event as No & Yes, respectively.(Index2=0,1, Index1=1) |

- Purpose: Set the configuration of server with multiple items.

| Syntax | http://{Server_IP}/auth/config.set?{argument1}&{argument2}&{..}&{THREAD_ID} |
| --- | --- |
| Parameters | The parameters in the configuration table. The space between words has to replaced by a “_”when entering a command, e.g. "Date Display Mode" should be entered as "Date_Display_Mode", and THREAD_ID must be the admin ID. |
| Example 1 | Set the Date Display Mode to "M/D/Y" mode |
|  | http://{ServerIP}/auth/config.set?Date_Display_Mode=1&THREAD_ID=ID Return: Success |

- Purpose: Get online user name and host name

| Syntax | http://{Server IP}/auth/linklist.get |
| --- | --- |
| Parameters | None |
| Example | Get online users information |
|  | http://{Server_IP}/auth/linklist.get Return: Host:192.168.7.174 connecting Level:user Host:192.168.8.201 connecting Level:(null) Host:192.168.8.201 connecting Level:(null) Host:192.168.8.201 connecting Level:(null) Host:192.168.7.180 connecting Level:admin. |

- Purpose: Kick out the current connections

| Syntax | http://{Server IP}/auth/linklist.set?{argument} |
| --- | --- |
| Parameters | KICKOUT_USER |
| Example | Kick out all connections |
|  | http://127.0.0.1/auth/linklist.set?KICKOUT_USER Return: Success |

- Purpose: Force to kick remote connection

| Syntax | http://{Server IP}/Forcekick.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | ITSELF, THREAD_ID |
| Example 1 | http://{ServerIP}/Forcekick.set?ITSELF&THREAD_ID=ID |

- Purpose: Set up remote burn or start remote burn

| Syntax | http://{Server IP}/auth/remoteBurn.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | ExportSetup, ExportStart, ExportClear, StartTime, EndTime, RealStartTime, RealEndTime, Idx1, THREAD_ID |
| Example 1 | Setup remote burn from start time to end time |
|  | http://{ServerIP}/auth/remoteBurn.set?ExportSetup& StartTime=TimeA&EndTime=TimeB&THREAD_ID=ID Where TimeA & TimeB represent the time info that contains the timezone_offset & DST in it. http://{ServerIP}/auth/remoteBurn.set?ExportSetup& RealStartTime=TimeC&RealEndTime=TimeD&THREAD_ID=ID Where TimeC & TimeD represent the pure time info that doesn’t have DST & timezone_offset in it. Return: DeviceName=[HLDTSTDVDRAM GH20NS10] ExportFormat=[DRV] Signature=[1] DiscErasing=[1] AvailSize=[4482 MB] If success, it returns the exported device( DeviceName ) we found, the exported format( ExportFormat ) we will used, will we do signature ( Signature ) and erasing disc ( DiscErasing ) and the available( AvailSize ) size we have. |
| Example 2 | To start the remote burn. |
|  | http://{ServerIP}/auth/remoteBurn.set?ExportStart&THREAD_ID=ID Return: If success, it starts to estimate required size and exporting if estimated result is good. |
| Example 3 | To clear/delete remote burn process |
|  | http://{ServerIP}/auth/remoteBurn.set?ExportClear&THREAD_ID=ID Return: success |
| Example 4 | Setup remote burn to export event index 100 |
|  | http://{ServerIP}/auth/remoteBurn.set?ExportSetup?Idx1=100&THREAD_ID=ID Return: DeviceName=[HLDTSTDVDRAM GH20NS10] ExportFormat=[DRV] Signature=[1] DiscErasing=[1] AvailSize=[4482 MB] If success, it returns the exported device( DeviceName ) we found, the exported format( ExportFormat ) we will used, will we do signature ( Signature ) and erasing disc ( DiscErasing ) and the available( AvailSize ) size we have. Note: The event index is the index query from event.get command [ http://{ServerIP}/event.get?START&THREAD_ID=ID ] |

- Purpose: Get remote burn status

| Syntax | http://{Server IP}/auth/remoteBurn.get?{argument} |
| --- | --- |
| Parameters | EstimateResult, ExportResult |
| Example 1 | To get the estimating result |
|  | http://{ServerIP}/auth/remoteBurn.get?EstimateResult Return: If export event video, the replied message will be… EstimateSize=[864 MB] EventCount=[1] If export normal video, the replied message will be … EstimateSize=[1021 MB] VideoStartTime=[ 1371637937] VideoEndTime=[ 1371638242] |
| Example 2 | To get the export status |
|  | http://{ServerIP}/auth/remoteBurn.get Return: The status of exporting and percentage we are now. |
| Example 3 | To get the last time exporting result |
|  | http://{ServerIP}/auth/remoteBurn.get?ExportResult Return: The result of last time we exporting, Success or Failed |

- Purpose: Set playback time and status

| Syntax | http://{Server IP}/Playback.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | START, STOP, PAUSE, RESUME, FastForward, FastBackward, StepForward, StepBackward, PBSpeed, STARTREAL, StartTime, EndTime |
| Example 1 | Start to playback with user time. [With timezone_offset & DST bias ] |
|  | http://{ServerIP}/Playback.set?START&StartTime= TimeA&THREAD_ID=ID TimeA is the time info that contains timezone_offset & DST bias in it. [user time] Return: SPEED=[> *1] success Note: Please send the user time which in the time range queried from info.get?PBTime. |
| Example 2 | Start to playback with real time. [Without timezone_offset & DST bias ] |
|  | http://{ServerIP}/Playback.set?STARTREAL&StartTime=TimeB &EndTime= TimeC&THREAD_ID=ID TimeB & TimeC are the pure time info that doesn’t contain timezone_offset & DST bias in it. [real time] Return: SPEED=[> *1] for PB success, SPEED=[-1] for PB failure. (The queried data doesn’t exist in the database.) Note: Please send the real time which in the time range queried from info.get?PBTime. |
| Example 3 | To set fast forward/backward speed. ( range from -6 ~ 5 ) arg; Speed. +5; +32X. +4; +16X. +3; +8X. +2; +4X. +1; +2X. 0; +1X. -1; -1X. -2; -2X. -3; -4X. -4; -8X. -5; -16X. -6; -32X. Note: (+) is for fast-forward, and (-) is for fast-backward. |
|  | Ex: http://{ServerIP}/Playback.set?PBSpeed=2&THREAD_ID=ID To setup playback speed as +4X fast-forward. |
| Example 4 | To pause. |
|  | http://{ServerIP}/Playback.set?PAUSE&THREAD_ID=ID |
| Example 5 | To resume the playback status. Often used after issuing PAUSE command. |
|  | http://{ServerIP}/Playback.set?RESUME&THREAD_ID=ID |
| Example 6 | To stop. |
|  | http://{ServerIP}/Playback.set?STOP&THREAD_ID=ID |
| Example 7 | To fast-forward. |
|  | http://{ServerIP}/Playback.set?FastForward&THREAD_ID=ID |
| Example 8 | To back-forward. |
|  | http://{ServerIP}/Playback.set?FastBackward&THREAD_ID=ID |
| Example 9 | To step-forward. |
|  | http://{ServerIP}/Playback.set?StepForward&THREAD_ID=ID |
| Example10 | To step-backward. |
|  | http://{ServerIP}/Playback.set?StepBackward&THREAD_ID=ID |
| Example11 | To perform multi-shoot. |
|  | Format: http://{ServerIP}/Playback.set?MS&CS=[00000000]&SS=[00]&ST=[xxxx]& ET=[xxxx]&ET=[XXXX]&THREAD_ID=ID MS : Stands for “Multi-Shoot”, must be the 1st parameter. CS : Stands for “Channel Selected”, Hex string, bit-wise mapping. No need to fill 0 on the head. - Ex: CS=[01] => Select Ch0, CS=[03] => Select Ch0 & Ch1 CS=[43] => Select Ch0, Ch1 & Ch6 SS : Stands for “Stream Selected”, Hex String, 1 bit for I stream type. No need to fill 0 on the head. - Ex: SS=[01] => Select 1st stream (aka. main stream) SS=[03] => Select 1st stream & 2nd stream (aks. main + dual stream) ST : Stands for “Start Time”, time convert to decimal digits. ET : Stands for “End Time”, time convert to decimal digits. - Just in case for the boundary control. RST: Stands for “Real Start Time”, time convert to decimal digits. - Time info without timezone offset & DST bias. RET: Stands for “Real End Time”, time convert to decimal digits. - Time info without timezone offset & DST bias. - Just in case for the boundary control. FI : Stands for “Frame Interval”, unit in second, unsigned int convert to decimal digits, must be greater than 0. FC : Stands for “Frame Count”, unsigned short convert to decimal digits, must be greater than 0. It represents the total frame count local-side sent in this multi-shoot process. |
|  | Ex: http://{ServerIP}/Playback.set?MS&CS=01&SS=01&ST=123433243& ET=123519643&FI=43200&FC=3&THREAD_ID=ID CS=01 & SS=01 - Get multi-shoot of Ch0 on the 1st stream (main stream). ST=123433243 & ET=123519643 - Start time is 123433243 & end time is 123519643 FI=43200 - Frame interval 43200 seconds. FC=03 - Frame count is 3. The local NVR will send out total 3 frames in the multi-shoot process. |

- Purpose: Select bandwidth as high, middle, low

| Syntax | http://{Server IP}/bandwidth.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | HI, MIDDLE, LOW |
| Example | Set the bandwidth to HI |
|  | http://{ServerIP}/bandwidth.set?HI&THREAD_ID=ID Return: Set bandwidth as high. |

Purpose: Show event list

| Syntax | http://{Server IP}/event.get?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | START, PRE ,NEXT |
| Example 1 | Get event list(1~120),NOTE:the older firmware may be 1~1000 S/W(DST) : 0=Summer , 1=Winter Type : A=Alarm , M=Motion , L=Loss |
|  | http://{ServerIP}/event.get?START&THREAD_ID=ID Return: S/W; No.; Date/Time; Channel; Index; Type. 1; 1; 1137008127; 2; 105; A. 1; 2; 1137008146; 2; 104; A. 1; 3; 1137008441; 2; 103; A. |
| Example 2 | Get previous page event list |
|  | http://{ServerIP}/event.get?PRE&THREAD_ID=ID Return: S/W; No.; Date/Time; Channel; Index; Type. 0; 1001; 1136912177; 8; 527; M. 0; 1002; 1136912352; 8; 526; M. 0; 1003; 1136912404; 8; 525; M. |
| Example 3 | Get next page event list |
|  | http://{ServerIP}/event.get?NEXT&THREAD_ID=ID Return: S/W; No.; Date/Time; Channel; Index; Type. 1; 1; 1137008127; 2; 105; A. 1; 2; 1137008146; 2; 104; A. |

- Purpose: Play event video

| Syntax | http://{Server IP}/event.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | Index |
| Example | Play event index=101 video |
|  | http://{ServerIP}/event.set?Index=101&THREAD_ID=ID Return: SPEED=[> *1] success |

- Purpose: In Live mode, get all channel data

| Syntax | http://{Server IP}/showch.set? ch{channel id}=2, 1 or 0 (1:enable main stream, 2: enable dual stream, 3:dual + main, 0:disable)&CO=xxx&{THREAD_ID} |
| --- | --- |
| Parameters | Channel ID,, Covert unblocking setting, THREAD ID |
| Example | Max channel =4 |
|  | http://{ServerIP}/ showch.set?ch1=1&ch2=1&ch3=1&ch4=1&CO=3&THREAD_ID=ID Return: - If succeed, it returns “set Success”. - If thread id is not registered, it returns “Not available THREAD_ID”. - The definition of CO argument is the same as video.vdo command |

- Purpose: When play event or playback video, get all channel data

| Syntax | http://{Server IP}/showpbch.set?ch{channel id}=2, 1 or 0 (1:enable main stream, 2: enable dual stream, 3: dual + main, 0:disable)&CO=xx&{THREAD_ID} |
| --- | --- |
| Parameters | Channel ID,, covert unblocking setting, THREAD ID |
| Example | Max channel =4 |
|  | http://{ServerIP}/ showpbch.set?ch1=1&ch2=1&ch3=1&ch4=1&CO=3&THREAD_ID=ID Return: 1. If succeed, it returns “set Success”. 2. If thread id is not registered, it returns “Not available THREAD_ID”. 3. The CO argument using is the same with video.vdo command |

- Purpose: Set the audio output

| Syntax | http://{Server IP}/audio.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | ON,OFF,FOLLOW,ALL |
| Example | Open audio |
|  | http://{Server_IP}/audio.set?ON&THREAD_ID=ID Return: Success |

- Purpose: Get the state of audio

| Syntax | http://{Server IP}/audio.get? {THREAD_ID} |
| --- | --- |
| Parameters | None. |
| Example | http://{Server_IP}/audio.get?&THREAD_ID=ID Return: Audio: [OFF] // current audio setting |

- Purpose:Set Open/Close two way audio [ DG600G2 DVR ONLY ]

| Syntax | http://{Server IP}/Talk.set?{argument}&{THREAD_ID} |
| --- | --- |
| Parameters | ON, OFF, THREAD ID |
| Example | Open two way audio |
|  | http://{Server_IP}/Talk.set?ON&THREAD_ID=ID Return: <p>TwoWayAudioPort=[port]</p>, two way audio server’s port at local NVR. <p>SetTalkON=[Success]</p> |

- Purpose:Set Relay Out On/Off

| Syntax | http://{Server IP}/RelayOut.set?RELAYOUT[idx]=OnOff&{THREAD_ID} |
| --- | --- |
| Parameters | RELAYOUT[idx] = OnOff; OnOff: 1 for ON, 0 for Off |
| Example | idx. 0; RelayA. 1; RelayB. 2; RelayC. Ex: http://{Server_IP}/RelayOut.set?RELAYOUT[0]=0&THREAD_ID=ID This command will turn off RelayA. Set as off Return : Success |

- Purpose: Obtain dome information

| Syntax | http://{Server IP}/dome.get |
| --- | --- |
| Parameters | None. |
| Example | Obtain dome information (1) DOME_PROTOCOL : DOME_PROTOCOL. 0; None. 1; DSCP. 2; AD422. 3; PELCO D. 4; PELCO P. 5; Fastrax 2. 6; JVC. 7; Panasonic_C. 8; Panasonic_N. 9; GE RS-485. 10; GE RS-422. (2) DOME_PROTOCOL_NAME : All dome protocol option from OSD |
|  | Ex: http://127.0.0.1/dome.get Return: DOME_PROTOCOL=[9,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,] DOME_PROTOCOL_NAME=[ None, DSCP, AD422, PELCO D, PELCO P, Fastrax 2, JVC, Panasonic_C, Panasonic_N, GE RS-485, GE RS-422,] DOME_ID=[0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,1,1,1,] |

- Purpose: Dome control

| Syntax | http://{Server IP}/dome.set?{argument}&CH=channel&DOME_ID=ID& DOME_PROTOCOL=D_P&PERCENTAGE=P&THREAD_ID=ID |
| --- | --- |
| Parameters | STOP, LEFT, RIGHT, UP, DOWN, ZOOM_OUT, ZOOM_IN,IRIS_OPEN,IRIS_CLOSE,FOCUS_FAR,FOCUS_NEAR, AUTO_FOCUS,SET_PRESET,GO_TOUR,AUTOPAN,CRUISE, DOME_MENU_ENTER, DOME_MENU_LEFT, DOME_MENU_RIGHT, DOME_MENU_UP, DOME_MENU_DOWN, THREAD_ID |
| Example | Control Dome command |
|  | http://{ServerIP}/dome.set?AUTO_FOCUS&CH=0&DOME_ID=0 &DOME_PROTOCOL=5&PERCENTAGE=100&THREAD_ID=ID |
| Syntax | http://{ServerIP}/dome.set?{argument=point}&CH=channel&DOME_ID=ID& DOME_PROTOCOL=D_P&THREAD_ID=ID |
| Parameters | GO_PRESET, SET_PRESET, Point=0~7 |
| Example | Control Dome command |
|  | http://{ServerIP}/dome.set?GO_PRESET=3&CH=0&DOME_ID=0 &DOME_PROTOCOL=5&THREAD_ID=ID |

- Purpose: Initial upgrade firmware

| Syntax | http://{Server IP}/auth/UploadUpgFile.set |
| --- | --- |
| Example | http://{ServerIP}/auth/UploadUpgFile.set User can get the max_packet_size from returned value. User need to limit the upload package size by this value |

- Purpose: Upgrade firmware file

| Syntax | http://{Server IP}/auth/upgrade.set |
| --- | --- |
| Example | http://127.0.0.1/auth/upgrade.set To divide file in some packages to upload one by one. |

- Purpose: Get the status during upgrading firmware

| Syntax | http://{Server IP}/UpgradeStatus.get?&THREAD_ID=ID |
| --- | --- |
| Example | http://{ServerIP}/UpgradeStatus.get?&THREAD_ID=ID You can get the returned string to descript the upgrading state. |

- Purpose: Upload config file

| Syntax | http://{Server IP}/auth/configUpload.set?&THREAD_ID=ID |
| --- | --- |
| Example | http://{Server IP}/auth/configUpload.set?&THREAD_ID=ID Return: User need to specify the config file they want to upload in the local PC. |

- Purpose: Import config file

| Syntax | http://{Server IP}/auth/importConfig.set |
| --- | --- |
| Example | http://{ServerIP}/auth/importConfig.set Return: After uploading, a text message is returned “Detected config changes that would require reboot”. |

- Purpose: Confirm to reboot NVR for importing config file

| Syntax | http://{Server IP}/auth/confirmImportConfig.set |
| --- | --- |
| Parameters | None. |
| Example | http://{ServerIP}/auth/confirmImportConfig.set Return: NVR reboots after issuing this command to apply the config file imported. |

- Purpose: Download current config file

| Syntax | http://{Server IP}/auth/configDownload.get |
| --- | --- |
| Parameters | None. |
| Example | http://{ServerIP}/auth/configDownload.get |

- Purpose: Get state of HDD

| Syntax | http://{Server IP}/StatusHDD.get?{THREAD_ID} |
| --- | --- |
| Parameters | THREAD_ID |
| Example 1 | http://{ServerIP}/StatusHdd.get? THREAD_ID=ID Return: It returns the status of the HDD installed in the NVR. HDD1=[ Device Name, Internal / External, Capacity, Active / Non-Active, Temperature ] Ex: HDD1=[I1:ATA ST3160815AS , Internal, 136GB, Active, 45] |

- Purpose: Get state of NAS

| Syntax | http://{Server IP}/ StatusHDD_DS200.get?{THREAD_ID} |
| --- | --- |
| Parameters | THREAD_ID |
| Example 1 | http://{ServerIP}/StatusHdd_DS200.get? THREAD_ID=ID |

- Purpose: Get state of record

| Syntax | http://{Server IP}/ RecordStatus.get?{THREAD_ID} |
| --- | --- |
| Parameters | THREAD_ID |
| Example 1 | http://{ServerIP}/RecordStatus.get? THREAD_ID=ID Return: Record Status=[ state string ] ,state string= [ Schedule Record, Event Only ] |

- Purpose: Get NVR system log

| Syntax | http://{Server IP}/auth/log.get |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{ServerIP}/auth/log.get Return: System Logs ID Date Time Code Value OFF 130618 202823 --MENU-- Calendar Search OFF 130618 201217 --MENU-- Account Setup OFF 130618 201215 --MENU-- User Management OFF 130618 201211 --MENU-- System Setup OFF 130618 200026 --MENU-- Analog Camera OFF 130618 200026 --MENU-- Camera Setup OFF 130618 192508 DISKACC 00000001 OFF 130618 192508 DISKPRE 00000001 OFF 130618 192508 DISKATT 00000001 SYSTEM 130618 192452 BOOT REBOOT OFF 130618 192324 SHUTDOWN REBOOT OFF 130618 192317 --MENU-- Monitor Setup OFF 130618 191956 DISKACC 00000001 |

- Purpose: Set DVR/NVR system log

| Syntax | http://{Server IP}/auth/log.set?{parameter} |
| --- | --- |
| Parameters | CLEAR_ALL |
| Example 1 | http://{ServerIP}/auth/log.set?CLEAR_ALL To delete all system log. Return: Success. |

- Purpose: Get IP camera list via UPNP search [ HDVR, NVR ONLY ]

| Syntax | http://{Server IP}/auth/upnp.get |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{ServerIP}/auth/upnp.get Return: If privilege is not high enough, “Access Deny!” is returned. Otherwise, return a list of IP cameras found in the network. UPnP List (total_cnt = 12) Idx; Brand; Model; IP. 1; ASUSTek; RT-N16; 192.168.5.1. 2; Panasonic_WV-SC385/ONVIF; WV-SC385; 192.168.5.138. 3; NV102; NV102; 192.168.5.150. 4; Samsung-Electronics; 1.0; 192.168.5.155. 5; unknown; unknown; 192.168.5.169. 6; CM-4221-01/Onvif; CM-4221-01; 192.168.5.172. 7; NA063/Onvif; NA063; 192.168.5.173. 8; NH073/Onvif; NH073; 192.168.5.175. 9; unknown; unknown; 192.168.5.177. 10; GeoVision/Onvif; GV-SD200; 192.168.5.189. 11; NV103/Onvif; NV103; 192.168.5.190. 12; HD30/Onvif; AvertX; 192.168.5.219. |

- Purpose: Get data protection list

| Syntax | http://{Server IP}/auth/dataprotection.get |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{ServerIP}/auth/dataprotection.get Return: StartFileNo; Date Time; Event; Loc.; Lock. 827321; 2011/11/16 20:41:54; YES; INT01; 0. 827392; 2011/11/16 20:55:29; YES; INT01; 0. 827648; 2011/11/16 21:45:42; YES; INT01; 0. If privilege is not enough, “Access Deny!” is returned. |

- Purpose: Show current activated account information

| Syntax | http://{Server IP}/auth/accountInfo.get&{THRTEAD_ID} |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{Server_IP}/auth/accountInfo.get?&THREAD_ID=ID Return: Admin & user1 is activated & the others are disabled. Account; Password; Level. admin; 1234; 8. user; 4321; 4. |

- Purpose: List account setting

| Syntax | http://{Server IP}/auth/account.get |
| --- | --- |
| Parameters | None. |
| Example 1 | Ex: http://{ServerIP}/auth/account.get Return: Account (length: 1~8): Account name. Password (length: 4~8): Account’s password. Privilege (0~8:0: Disable, 1~8: level1~8): Account’s privilege level. Account; Password; Privilege. admin; 1234; 8. user; 4321; 4. user2; 4321; 0. user3; 4321; 0. user4; 4321; 0. user5; 4321; 0. user6; 4321; 0. user7; 4321; 0. |

- Purpose: List permission level for each function/menu

| Syntax | http://{Server IP}/auth/permissions.get |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{Server_IP}/auth/permissions.get Return: Function: function/menu name Permission: function/menu’s privilege level. - Range from 0 to 8; 0 for None,1~8 for level1 to level8. Function; Permission. Playback/Search; 0. Covert Camera; 8. Dome Control; 0. Live Operation; 0. Call Control; 0. Export Data; 0. Menu Access; 4. System Setup; 8. Monitor Setup; 8. Camera Setup; 8. Record Setup; 8. Sequence Setup; 8. Event Setup; 8. Database Setup; 8. Configuration; 8. AUX Setup; 8. Shutdown; 8. |

- Purpose: List current PoE status [ HDVR, NVR ONLY ]

| Syntax | http://{Server IP}/auth/poe.get |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{Server_IP}/auth/poe.get return: Port: Port number. Link: Link status. Consumption: power consumption Mac: MAC address of the IP device. Port; Link; Consumption[W]; Mac. 01; up; 4.9 W; 00:D0:89:08:37:CF. 02; down; 0.0 W; --:--:--:--:--:--. 03; up; 5.2 W; 00:D0:89:08:37:D5. 04; up; 3.4 W; 00:D0:89:08:AA:01. 05; up; 3.5 W; 00:D0:89:08:37:09. 06; up; 3.4 W; 00:01:02:03:04:05. 07; up; 5.5 W; 60:9A:A4:00:09:46. 08; up; 5.3 W; 60:9A:A4:00:09:48. |

- Purpose: List camera status [ Only IP cam info can be shown ] [ HDVR, NVR ONLY ]

| Syntax | http://{Server IP}/auth/caminfo.get |
| --- | --- |
| Parameters | None. |
| Example 1 | http://{Server_IP}/auth/caminfo.get return: CH:Which Channel Format: video format Resolution: Video Resolution PPS: Video frame rate per second Bandwidth: Video stream size per second. CH; Format; Resolution; PPS; Bandwidth. CH01-Main; H264(MP); 1080p; 25.00; 521.94 KB/Sec. CH02-Main; ----; ----; ----; ----. CH03-Main; H264(MP); 1080p; 25.00; 508.90 KB/Sec. CH04-Main; H264(MP); 1080p; 25.00; 507.32 KB/Sec. CH05-Main; H264(MP); 1280x720; 30.00; 168.48 KB/Sec. CH06-Main; H264(MP); 1280x720; 24.80; 181.66 KB/Sec. CH07-Main; H264(BL); 1280x720; 15.00; 511.04 KB/Sec. CH08-Main; H264(BL); 1280x720; 15.00; 507.49 KB/Sec. CH01-Dual; H264(MP); 720x576; 24.80; 123.85 KB/Sec. CH02-Dual; ----; ----; ----; ----. CH03-Dual; H264(MP); 720x576; 25.00; 126.68 KB/Sec. CH04-Dual; H264(MP); 720x576; 25.00; 123.60 KB/Sec. CH05-Dual; H264(MP); 720x480; 30.00; 49.91 KB/Sec. CH06-Dual; H264(MP); 720x576; 24.80; 49.65 KB/Sec. CH07-Dual; H264(BL); 720x480; 30.00; 128.32 KB/Sec. CH08-Dual; H264(BL); 720x480; 30.00; 128.14 KB/Sec. |

- Purpose: Get calendar information

| Syntax | http://{Server IP}/auth/calendar.get?time=XXX |
| --- | --- |
| Parameters | time |
| Example 1 | http://{ServerIP}/auth/calendar.get?time=1371513600 Return: New Software Behavior: NVR returns the record data channel by channel. calendar.behavior = 1 is used to judge whether it’s new behavior or not. And below is its reply message. calendar.behavior = 1 calendar.time = 1371513600 calendar.day_list = 00 00 00 00 00 00 00 00 00 00 00 01 01 01 01 01 01 01 01 00 01 01 01 01 02 00 00 00 00 00 00 calendar.time_list [01] = 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 05 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 … calendar.time_list [20] = 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 calendar.behavior : Always equals to 1, it means the new calendar behavior. - New behavior: the recorded data can be separated channel by channel. calendar.time : Query time ( Represented in long int ) calendar.day_list : Lists all days in one month. (maximum 31 days) 00 : represent no data recorded in this day 01 : represent data recorded in this day. calendar.time_list [ channel_idx ] : Lists the record status of the specified day as given by calendar.time. And it’s separated in 10 minute intervals. So there are 144 (10-minute) intervals in one day. 00: no recorded data. 01: normal data. 03: normal data & alarm event. 05: normal data & motion event. 0A: normal data & video loss alarm. ======================================================= Old Software Behavior: Only returns the recorded data in a day. Not separated for each channel. And below is its reply message. calendar.time=1234567890 calendar.day_list=00 00 00 00 00 00 00 00 00 00 00 01 01 01 01 01 01 01 01 00 01 01 01 01 02 00 00 00 00 00 00 calendar.time_list=01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 calendar.time : Query time ( Represented in long int ) calendar.day_list : Lists all days in one month. (maximum 31 days) 00 : represent no data recorded in this day 01 : represent data recorded in this day. calendar.time_list= lists the record status of the specified day as given by calendar.time, broken up in 10 minute intervals. So there are 144 (10-minute) intervals in one day. (time_list[ ] & 0x11) : if not zero then it has normal (time_list[ ] & 0x22 ): if not zero then it has event |

- Purpose: Get Single image data

| Syntax | http://{Server IP}/ivop.get&{parameter} |
| --- | --- |
| Parameters | action, account, password, key |
| Example 1 | Login with auth/login http://{ServerIP}/ivop.get&action=live&THREAD_ID=ID Return: In live mode, an I-frame image of current recording format is returned. |
| Example 2 | Login with mlogin.get: http://{ServerIP}/ivop.get&account=user&password=passwd&key=key_id &THREAD_ID=ID Return: If JPEG encode is supported by NVR, a JPEG image html would be returned. If not supported, “<HTML><TITLE>Function Unsupported</TITLE><BODY>\r\n <H2>Dual streaming is required for HTML viewing</H2>\r\n </BODY></HTML>\r\n\r\n” is returned. NOTE: The key follows the HMAC-Digest standard. You can refer to the md5.js file by http://ip:port/md5.js. |